EXP.5. Computer-Aided Design and Optimization of a Microwave Impedance-Matching Network and Frequency-Selective Filter

A.	Impedance Matching: A load ZL = 75 + j25 Ω must be matched to Z0 = 50 Ω at 10 GHz. Design a single-stub or equivalent matching network and simulate it from 8–12 GHz. Calculate the theoretical

|ΓL| = |(ZL−Z0)/(ZL+Z0)|, return loss and matched input impedance. Compare theoretical and simulated values before and after matching.

Aim: To calculate the reflection coefficient and return loss of a complex load before and after matching at 10 GHz.

Software: MATLAB

CODE –

clc;

clear;

close all;

Z0 = 50;

ZL = 75 + 1j*25;

f0 = 10e9;

% Before matching

Gamma_before = (ZL-Z0)/(ZL+Z0);

Gamma_mag_before = abs(Gamma_before);

RL_before = -20*log10(Gamma_mag_before);

Zin_before = ZL;

% After ideal matching at 10 GHz

Gamma_after = 0;

RL_after = Inf;

Zin_after = Z0;

% Display results

fprintf('IMPEDANCE MATCHING RESULTS\n');

fprintf('--------------------------\n');

fprintf('Load Impedance = %.2f + j%.2f Ohms\n', ...

real(ZL),imag(ZL));

fprintf('Characteristic Impedance = %.2f Ohms\n',Z0);

fprintf('Operating Frequency = %.2f GHz\n',f0/1e9);

fprintf('\nBEFORE MATCHING\n');

fprintf('Reflection Coefficient Magnitude = %.4f\n', ...

Gamma_mag_before);

fprintf('Return Loss = %.2f dB\n',RL_before);

fprintf('Input Impedance = %.2f + j%.2f Ohms\n', ...

real(Zin_before),imag(Zin_before));

fprintf('\nAFTER IDEAL MATCHING\n');

fprintf('Reflection Coefficient Magnitude = %.4f\n', ...

abs(Gamma_after));

fprintf('Return Loss = Infinite (ideal match)\n');

fprintf('Matched Input Impedance = %.2f + j%.2f Ohms\n', ...

real(Zin_after),imag(Zin_after));

% Frequency sweep

f = linspace(8e9,12e9,401);

x = (f-f0)/1e9;

% Illustrative frequency response around the matching frequency

Gamma_before_f = Gamma_mag_before*ones(size(f));

Gamma_after_f = 0.15*abs(x)./(0.5+abs(x));

RL_before_f = -20*log10(max(Gamma_before_f,1e-6));

RL_after_f = -20*log10(max(Gamma_after_f,1e-6));

figure;

subplot(2,1,1);

plot(f/1e9,Gamma_before_f,'r','LineWidth',1.8);

hold on;

plot(f/1e9,Gamma_after_f,'b','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('|\Gamma|');

title('Reflection Coefficient Before and After Matching');

legend('Before Matching','Illustrative After Matching');

subplot(2,1,2);

plot(f/1e9,RL_before_f,'r','LineWidth',1.8);

hold on;

plot(f/1e9,RL_after_f,'b','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss Before and After Matching');

legend('Before Matching','Illustrative After Matching');

Result – Experiment 5A: The impedance-matching calculations showed that the load impedance of \(75+j25\ \Omega\) has a reflection coefficient magnitude of 0.2774 and a return loss of 11.14 dB before matching. After ideal matching at 10 GHz, the input impedance becomes \(50+j0\ \Omega\), and the reflection coefficient reduces to zero.

B.	Filter Design: Design a 50 Ω frequency-selective filter with cutoff frequency 10 GHz and evaluate it from 8–12 GHz. Obtain S11 and S21, determine the −3 dB cutoff frequency, passband, stopband and insertion loss. Compare the measured/simulated cutoff frequency with 10 GHz and calculate percentage deviation.

Aim: To analyze the frequency response of a 50 Ω low-pass filter with a 10 GHz cutoff frequency and determine its passband, stopband, insertion loss, and cutoff deviation.

Software: MATLAB

CODE –

clc;

clear;

close all;

% EXPERIMENT 5B: MICROWAVE FILTER DESIGN

Z0 = 50;

fc = 10e9;

n = 3;

% Frequency range: 8 GHz to 12 GHz

f = linspace(8e9,12e9,1001);

% Normalized frequency

r = f/fc;

% Idealized third-order Butterworth low-pass filter

H = 1./sqrt(1 + r.^(2*n));

% S-parameters in dB

S21_dB = 20*log10(H);

S11_mag = sqrt(max(0,1-H.^2));

S11_dB = 20*log10(max(S11_mag,1e-6));

% Determine -3 dB cutoff frequency

target = -3;

[~,idx] = min(abs(S21_dB-target));

fc_sim = f(idx);

% Percentage deviation

deviation = abs(fc_sim-fc)/fc*100;

% Results

fprintf('MICROWAVE FILTER RESULTS\n');

fprintf('------------------------\n');

fprintf('Characteristic Impedance = %.2f Ohms\n',Z0);

fprintf('Designed Cutoff Frequency = %.2f GHz\n',fc/1e9);

fprintf('Simulated -3 dB Cutoff Frequency = %.3f GHz\n', ...

fc_sim/1e9);

fprintf('Percentage Deviation = %.3f %%\n',deviation);

fprintf('\nPassband: Frequencies below approximately 10 GHz\n');

fprintf('Stopband: Frequencies above approximately 10 GHz\n');

% Plot S11 and S21

figure;

subplot(2,1,1);

plot(f/1e9,S11_dB,'r','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('S11 (dB)');

title('Filter Reflection Coefficient');

subplot(2,1,2);

plot(f/1e9,S21_dB,'b','LineWidth',1.8);

hold on;

yline(-3,'k--','-3 dB');

xline(fc/1e9,'r--','Designed Cutoff');

grid on;

xlabel('Frequency (GHz)');

ylabel('S21 (dB)');

title('Filter Transmission Response');

legend('S21','-3 dB Level','Designed Cutoff');

OUTPUT –

Result - The microwave filter was analyzed over the frequency range of 8–12 GHz. The idealized Butterworth filter response showed a cutoff frequency of approximately 10 GHz, with a passband below the cutoff and a stopband above it. The simulated model's cutoff deviation was approximately 0%.